(5pts) Given the following registers and snippet of memory, what does each move instruction store to register %rax?

| %rdi | 0x800 |
| --- | --- |
| %rsi | 0x5 |
| %rdx | 0x8040600 |

movq	%rdi,	%rax

movq	(%rdx), %rax

| 8-byte values | addresses |
| --- | --- |
| 0x004 | 0x8040618 |
| 0x003 | 0x8040610 |
| 0x070 | 0x8040608 |
| 0x0A0 | 0x8040600 |
| 0x300 | 0x80405f8 |
| 0x005 | 0x80405f0 |

movq	0x6(%rdx, %rsi,2), %rax

Can you decode what the following two sets of instructions are doing

- subq $8, %rsp

movq %rbp, (%rsp)

- movq (%rsp), %rax

addq $8, %rsp